Unit – 2

IoT Protocols

Protocol Standardization for IoT

• IoT-Architecture one of the few efforts targeting  a holistic architecture for all IoT sectors • This consortium consists of 17 European  organizations from nine countries

• Summarized current status of IoT standardization  as

• Fragmented architectures

• No holistic approach to implement IoT has yet been  proposed

• Many island solutions do exist (RFID, sensor nets, etc.)

• Little cross-sector reuse of technology and exchange of  knowledge

M2M and WSN Protocols

• Most M2M applications are developed today in a  highly customized fashion

• High-level M2M architecture from M2M  Standardization Task Force (MSTF) does include  fixed & other non cellular wireless networks

• Means it’s generic, holistic IoT architecture even  though it is M2M architecture

• M2M and IoT sometimes are used interchangeably  in the United States.

M2M and WSN Protocols

• Other M2M standards activities include: • Data transport protocol standards - M2MXML,  JavaScript Object Notation (JSON), BiTXML, WMMP,  MDMP

• Extend OMA DM to support M2M devices protocol  management objects

• M2M device management, standardize M2M gateway • M2M security and fraud detection

• Network API’s M2M service capabilities

• Remote management of device behind gateway/firewall • Open REST-based API for M2M applications

SCADA and RFID Protocols

• Supervisory Control And Data Acquisition • One of the IoT pillars to represent the whole  industrial automation arena

• IEEE created standard specification called Std C37.1™, for SCADA & automation systems in 2007 • In recent years, network-based industrial  automation has greatly evolved

• With the use of intelligent electronic devices (IEDs),  or IoT devices in our terms, in substations and  power stations

SCADA and RFID Protocols

SCADA and RFID Protocols

• The processing is now distributed

• Functions that used to be done at control center  can now be done by IED i.e. M2M between devices

• Due to restructuring of electric industry, traditional  vertically integrated electric utilities are replaced  by many entities such as

• GENCO (Generation Company),

• TRANSCO (Transmission Company),

• DISCO (Distribution Company),

• ISO (Independent System Operator), etc.

Issues with IoT Standardization

• It should be noted that not everything about  standardization is positive

• Standardization is like a double-edged sword: • Critical to market development

• But it may threaten innovation and inhibit change when  standards are accepted by the market

• Standardization and innovation are like yin & yang

Issues with IoT Standardization

• They could be contradictory to each other in some  cases, even though this observation is debatable

• Different consortia, forums and alliances have  been doing standardization in their own limited  scope

• For example, 3GPP covers only cellular wireless  networks while EPCglobal’s middleware covers  only RFID events

Issues with IoT Standardization

• Even within same segment, there are more than  one consortium or forum doing standardization  without enough communication with each other

• Some are even competing with each other

• Some people believe that the IoT concept is well  established

Issues with IoT Standardization

• However, some gray zones remain in the  definition, especially which technology should be  included

• Following two issues for IoT standardization in  particular and ICT standardization in general may  never have answers:

1. ICT standardization is a highly decentralized  activity. How can the individual activities of the

Issues with IoT Standardization

network of extremely heterogeneous

standardssetting bodies be coordinated?

2. It will become essential to allow all interested  stakeholders to participate in the

standardization process toward the IoT and to  voice their respective requirements and  concerns. How can this be achieved?

Unified Data Standards

• Already discussed about two pillars of the Internet

• HTML/HTTP combination of data format and  exchange protocol is the foundation pillar of WWW

• Described great number of data standards and  protocols proposed for four pillar domains of IoT

• Many issues still impede the development of IoT  and especially WoT vision

Unified Data Standards

Evolution of web

Unified Data Standards

• Many standardization efforts have been trying to  define unified data representation, protocol for IoT

• Before IoT, Internet was actually an Internet of  documents or of multimedia documents

• Two pillars of Internet including HTML/HTTP turned  the Internet into WWW

• We need to turn the IoT into the WoT

• What will it take to make this to happen?

• Do we need a new HTML/HTTP-like standard for  MTC and WoT? If there is no need to reinvent the

Unified Data Standards

wheel, what extensions do we need to build on top  of HTML/HTTP or HTML5?

• Browser is intended for humans, so do we need  new browser for machines to make sense of ocean  of machine-generated data? If not, what extensions  do we need to make to the existing browsers?

• Today, most new protocols are built on top of XML.  For OS there must be XML-based data format  standards or a metadata standard to represent the  machine-generated data (MGD). Is it possible to  define such a metadata standard that covers  everything?

Unified Data Standards

• There are many different levels of protocols

• But the ones that most directly relate to business  and social issues are the ones closest to the top

• so-called application protocols such as HTML/HTTP  for the web

• Web has always been visual medium, but restricted

• Until recently, HTML developers were limited to  CSS

& JavaScript in order to produce animations • Or they would have to rely on a plug-in like Flash

Unified Data Standards

Protocols – IEEE 802.15.4

• Defines operation of low-rate wireless personal  area networks (LR-WPANs)

• Specifies physical layer and media access control  for LR-WPANs

• Maintained by IEEE 802.15 working group, which  defined the standard in 2003

• Basic framework conceives a 10m communications  range with a transfer rate of 250 kbit/s

Protocols – IEEE 802.15.4

• Physical Layer (PHY)

provides data transmission

service & interface to

physical layer management

entity

• MAC enables transmission

of MAC frames through the

use of the physical channel

BACNet Protocol

• Communications protocol for Building Automation and  Control (BAC) networks

• Provides mechanisms for computerized building  automation devices to exchange information

• Designed to allow communication of building  automation & control system for application like • Heating, Ventilating and Air-conditioning Control (HVAC) • Lighting Control, Access Control

• Fire Detection Systems and their Associated Equipment

BACNet Protocol

• Defines a number of services that are used to  communicate between building devices

• Protocol services include Who-Is, I-Am, Who-Has,  IHave which are used for Device & Object discovery

• Services such as Read-Property and Write-Property  are used for data sharing

• Defines 60 object types that are acted upon by  services

• Defines no. of data link/physical layers including • ARCNET,

• Ethernet,

• BACnet/IP,

• BACnet/IPv6,

• Point-To-Point over RS-232,

• Master-Slave/Token-Passing over RS-485,  • ZigBee

• LonTalk

Modbus

• Serial communications protocol originally published  by Modicon (now Schneider Electric) in 1979

• Commonly available for connecting industrial  electronic devices

• Reasons for use of Modbus in industrial  environment:

• Developed with industrial applications in mind • Openly published and royalty-free

• Easy to deploy and maintain

• Enables communication among many devices  connected to the same network

Modbus Object Types

| Object type | Access | Size |
| --- | --- | --- |
| Coil | Read-write | 1 bit |
| Discrete input | Read-only | 1 bit |
| Input register | Read-only | 16 bits |
| Holding register | Read-write | 16 bits |

Protocol Versions

• Modbus RTU

• Modbus ASCII

• Modbus TCP/IP or Modbus TCP

• Modbus over TCP/IP or Modbus over TCP or  Modbus RTU/IP

• Modbus over UDP

• Modbus Plus (Modbus+, MB+ or MBP) • Pemex Modbus

• Enron Modbus

KNX Protocol

• Standardized (EN 50090, ISO/IEC 14543), OSI-based  network communications protocol for automation

• Defines several physical communication media: • Twisted pair wiring (inherited from the BatiBUS and EIB  Instabus standards)

• Powerline networking (inherited from EIB and EHS similar to  that used by X10)

• Radio (KNX-RF)

• Infrared

• Ethernet (also known as EIBnet/IP or KNXnet/IP)

KNX System Components

• All the devices for a KNX installation are connected  together by a two wire bus to exchange data

• Sensors

• Actuators

• System devices and components

ZigBee

• IEEE 802.15.4-based specification for a suite of  high-level communication protocols

• Used to create personal area networks with  small, low-power digital radios

• ZigBee based applications

• Home Automation

• Medical Device Data Collection

• other low-power low-bandwidth

ZigBee Architecture

ZigBee Architecture

• Divided into three sections

• IEEE 802.15.4 which consists of MAC and physical layers • ZigBee layers, which consist of the network layer, the  ZigBee device object (ZDO), the application sublayer,  and security management

• Manufacturer application: Manufacturers of ZigBee  devices can use the ZigBee application profile or  develop their own application profile

Network Layer

• Located between the MAC layer and application  support sublayer

• Provides the following functions:

• Starting a network

• Managing end devices joining or leaving a network • Route discovery

• Neighbor discovery

APS Layer

• Application Support Sublayer (APS)

• Provides services necessary for application objects  (endpoints) and the ZigBee device object (ZDO)

• Some of services provided by the APS to the  application objects for data transfer are • Request

• Confirm

• Response

APS Layer

• Application Object

(endpoint)

• Defines input and output to

the APS

• For example, a switch that

controls a light is the input

from the application object,

and the output is the light

bulb condition

• Each node can have 240 separate application objects

APS Layer

• ZigBee Device Object (ZDO)

• Control and management of application objects •  Performs overall device management tasks: • Determines the type of device in a network (for example, end  device, router, or coordinator)

• Initializes the APS, network layer, and security service provider • Performs device and service discovery

• Initializes coordinator for establishing a network • Security management

• Network management

APS Layer

• End Node

• Each end node or end device can have multiple EPs • Each EP contains an application profile, such as home  automation

• can be used to control multiple devices or single device • ZigBee Addressing Mode

• ZigBee uses direct, group, and broadcast addressing for  transmission of information

Security Requirements

IOT Security

• Fundamental idea - IoT will connect all objects  around us to provide smooth communication • Economic of scale in IoT presents new security

challenges for global devices in terms of – Authentication

– Addressing

– Embedded Security

Security Requirements

IOT Security

• Devices like RFID and sensor nodes have no  access control functionality

• Can freely obtain or exchange information from  each other

• So authentication & authorization scheme must  be established between these devices to  achieve the security goals for IoT

Security Requirements

• Privacy of things and security of data is one of  the key challenges in the IoT

• Unauthorized Access

– One of the main threats is the tampering of  resources by unauthorized access

– Identity-based verification should be done before  granting the access rights

• Information corruption

Security Requirements

– Device credential must be protected from  tampering

– Secure design of access rights, credential and  exchange is required to avoid corruption

• Theft of Resources

– Access of shared resources over insecure channel  causes theft of resources

– Results into man-in-the-middle attack

• Information Disclosure

– Data is stored at different places in different forms

Security Requirements

– Distributed data must be protected from disclosure – Context-aware access control must be enforced to  regulate access to system resources

• DoS Attack

– Denial of Service (DoS)

– Makes an attempt to prevent authentic user from  accessing services which they are eligible for – For example, unauthorized user sends to many  requests to server

Security Requirements Access

Scalabilit y

Tamper  Resistan ce

Flexibilit y &

Adaptabi lity

Secure  Storage

Control

Authenti

cation

Data

Confiden

tiality

Security

Requireme

nts

Availabili

ty

Trust

Manage

ment Secure

Software

Executio

n

Security Requirements

• Access Control

– Provides authorized access to network resources – IoT is ad-hoc, and dynamic in nature

– Efficient & robust mechanism of secure access to  resources must be deployed with distributed  nature

• Authentication

– Identity establishment b/w communicating devices

Security Requirements

– Due to diversity of devices & end users, an attack  resistant and lightweight solution for

authentication

• Data Confidentiality

– Protecting data from unauthorized disclosure – Secure, lightweight, and efficient key exchange  mechanism is required

• Availability

Security Requirements

– Ensuring no denial of authorized access to network  resources

• Trust Management

– Decision rules needs to be evolved for trust  management in IoT

• Secure Software Execution

– Secure, managed-code, runtime environment  designed to protect against different applications

• Secure Storage

Security Requirements

– Involves confidentiality and integrity of sensitive  information stored in the system

• Tamper Resistance

– Desire to maintain security requirements even  when device falls into hands of malicious parties – Can be physically or logically probed

• Scalability

– IoT consist of various types of devices with  different capabilities from intelligent sensors and  actuators, to home appliances

Security Requirements

– Communication (wire or wireless) & protocols  (Bluetooth, ZigBee, RFID, Wi-Fi, etc.)

• Flexibility and Adaptability

– IoT will consist of mobile communication devices – Can roam around freely from one type of  environment to others

– With different type of risks and security threats – So users are likely to have different privacy profile  depending on environment

Security Architecture for IoT

Threat Modeling

• Presented by first defining misuse case • Means negative scenario describing the ways  the system should not work

• And then standard use case

• Assets to be protected in IoT will vary with  respect to every scenario case

Threat Analysis

• Assets needs to be identified to drive threat  analysis process

• Smart home is localized in space, provide  services in a household

• Devices in Smart Home are combined with n/w • Provide means for entertainment, monitoring  of appliances, controlling of house components  and other services

Use Cases and Misuse Cases

• Actor in use case and misuse case in the  scenario of smart home includes:

– Infrastructure owner (smart home)

– IoT entity (smartphone device or software agent) – Attacker (misuser)

– Intruder (exploiter)

Use Cases and Misuse Cases

• Access rights granted to unauthorized entity • Corruption of access credentials

• Unauthorized data transmission

• Denial of service (DoS) attack

• Man-in-the-middle attack

IoT Security Tomography

• Classified according to attacks addressing to  different layers

– Transport Layer

– Network Layer

– MAC layer

– RF layer

IoT Security Tomography

Key Elements of Security

• Authentication

• Access Control

• Data and Message Security

• Prevention from denial of taking part in a  transaction

Identity Establishment

• Secure Entity Identification or Authentication • Authentication is identity establishment  between communicating devices or entities • Entity can be a single user, a set of users, an  entire organization or some networking device • Identity establishment is ensuring that origin of  electronic document & message is correctly  identified.

Access Control

• Also known as access authorization • Principles is to determine who should be able  to access what

• Prevents unauthorized use of resources • To achieve access control, entity which trying  to gain access must be authenticated first • According to authentication, access rights can  be modified to the individual

Data and Message Security

• Related with source authenticity, modification  detection and confidentiality of data

• Combination of modification & confidentiality  of message is not enough for data integrity • But origin of authenticity is also important • Location privacy is equally important risk in IoT • Should not be any way for attacker to reveal  identity or location information of device

Security Model for IoT